##### REMOTE CONTROLLER WITH FIVE FUNCTIONS

##### Features

- Wide operating voltage range: 2.4V to 5.0V

- Low Standby current

- Auto Power-OFF function for TX2

- Few external components needed

##### General Description

The TX2/RX2 are a pair of CMOS LSIs designed for remote controlled car applications.  TX2 is  built with auto Power-OFF function.  The TX2/RX2 has five control keys controlling the motions (i.e. Forward, Backward, Rightward, Leftward and Turbo function) of the remote controlled car.

Absolute Maximum Ratings							Comments*

DC Supply Voltage.....................................0.3V to 5.0V

Input/Output Voltage...........GND -0.2V to VDD + 0.2V

Operating temperature..............................-10C to 60C

Storage Temperature..............................-25C to 125C

Never allow a stress to exceed the values listed under "Absolute Maximun Ratings", otherwise the device would suffer from a permanent damage. Nor is a stress at the listed value be allowed to persist over a period, since an extended exposure to the absolute maximum rating condition may also affect the reliability of the device, if not causing a damage thereof.

##### Electrical Characteristics

TX2

(VDD=4.5V, Fosc = 128KHz, TA=25C, unless otherwise specified.)

| Parameter | Symbol | Min. | Typ. | Max. |
| --- | --- | --- | --- | --- |
| Operating Voltage | VDD | 2.4V | 4.0V | 5.0V |
| Operating Current | Idd | - | - | 1mA |
| Stand-by Current | Istb | - | - | 1A |
| DC O/P Driving Current | Idrive | 5mA | - | - |
| AC O/P Driving Current | Idrive | 5mA | - | - |
| AC O/P Frequency | Faudio | 500Hz | - | 1KHz |

RX2

(VDD=4.0V, Fosc = 128KHz, TA=25C, unless otherwise specified.)

| Parameter | Symbol | Min. | Typ. | Max. |
| --- | --- | --- | --- | --- |
| Operating Voltage | VDD | 2.4V | 4.0V | 5.0V |
| Operating Current | Idd | - | - | 1mA |
| O/P Driving Current | Idrive | 1mA | - | - |
| O/P Sinking Current | Isink | 1mA | - | - |
| Effect Decoding Frequency Variation | Ftolerance | -20%zHzh jjjjjeiadkv | - | 20% |

##### Pin Description

TX2

| Pin No. | Designation | Description |
| --- | --- | --- |
| 1 | RIGHTB | The rightward function will be selected when this pin is connected to GND. |
| 2 | TESTB | This pin is used for testing purpose only. |
| 3 | GND | Negative power supply |
| 4 | BACKWARDB | The backward function will be selected when this pin is connected to GND. |
| 5 | FORWARDB | The forward function will be selected when this pin is connected to GND. |
| 6 | TURBOB | The turbo function will be selected when this pin is connected to GND. |
| 7 | SC | Output pin of the encoding signal with carrier frequency |
| 8 | SO | Output pin of the encoding signal without carrier frequency |
| 9 | VDD | Positive power supply |
| 10 | PC | Power control output pin |
| 11 | OSCO | Oscillator output pin |
| 12 | OSCI | Oscillator input pin |
| 13 | FOSC | This pin is used for testing mode. |
| 14 | LEFTB | The leftward function will be selected when this pin is connected to GND. |

RX2

| Pin No. | Designation | Description |
| --- | --- | --- |
| 1 | VO2 | Inverter 2 output pin for power amplify |
| 2 | GND | Negative power supply |
| 3 | SI | Input pin of the encoding signal |
| 4 | OSCI | Oscillator input pin |
| 5 | OSCO | Oscillator output pin |
| 6 | RIGHT | Rightward output pin |
| 7 | LEFT | Leftward output pin |
| 8 | RDB | Rightward function is disabled when this pin is connected to GND. |
| 9 | LDB | Leftward function is disabled when this pin is connected to GND. |
| 10 | BACKWARD | Backward output pin |
| 11 | FORWARD | Forward output pin |
| 12 | TURBO | TURBO output pin |
| 13 | VDD | Positive power supply |
| 14 | VI1 | Inverter 1 input pin for power amplify |
| 15 | VO1 | Inverter 1 output pin for power amplify |
| 16 | VI2 | Inverter 2 input pin for power amplify |

##### Code Format

Encode Rule

(A) Bit Format

##### Data Format

W2 W2 W2 W2 (n) x W1 W2 W2 W2 W2 (n) x W1 W2 W2 W2 W2

| Number of Function Codes (n) W1 | Function Key | Decode Result |
| --- | --- | --- |
| 4 |  | End Code |
| 10 | Forward | Forward |
| 16 | Forward & Turbo | Forward |
| 22 | Turbo | Turbo |
| 28 | Turbo & Forward & Left | Forward & Left |
| 34 | Turbo & Forward & Right | Forward & Right |
| 40 | Backward | Backward |
| 46 | Backward & Right | Backward & Right |
| 52 | Backward & Left | Backward & Left |
| 58 | Left | Left |
| 64 | Right | Right |

Encode/Decode Timing

(A)

(B)

##### Typical Application Circuit

Transmitter (TX2 Fosc  128 KHz)

Receiver (RX2 Fosc  128 KHz)

Infrared Application Circuit

Transmitter

Recevier

##### Testing Circuit

Fosc for TX2 Fosc, RX2 z

##### Bonding Diagram

TX2

RX2

Notice: REALTEK's products are sold by description only, REALTEK reserves the rights to make changes in circuit design and/or specification at any time

without notice. Accordingly, the reader is cautioned to make sure the latest version of data sheets are available before placing orders.

REALTEK Semiconductor Corp.

OFFICE:

3F, NO. 56 WU-KUNG 6 RD.,

WU-KU INDUSTRIAL PARK,

TAIPEI HSIEN, TAIWAN, R.O.C.

TEL:886-2-2980098

FAX:886-2-2980097

HEAD OFFICE:

1F, NO. 11, INDUSTRY E. RD. IX,

SCIENCE-BASED INDUSTRIAL PARK,

HSINCHU, 300 TAIWAN, R.O.C.

TEL:886-3-5780211

FAX:886-3-5776047