### What you should know.

- After studying this section, you should:

- • Know the EIA and military standard colour codes for inductors.

- • Be able to calculate the value of colour coded inductors.

## Fig 3.4.1 Four Band Standard EIA Colour Code For Inductors.

| Band | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Meaning | 1st Digit | 2nd Digit | Multiplier (No. of zeros) | Tolerance % |
| Gold |  |  | x 0.1 (divide by 10) | +/-5% |
| Silver |  |  | x 0.01 (divide by 100) | +/-10% |
| Black | 0 | 0 | x1 (No Zeros) | +/-20% |
| Brown | 1 | 1 | x10 (0) |  |
| Red | 2 | 2 | x100 (00) |  |
| Orange | 3 | 3 | x1000 (000) |  |
| Yellow | 4 | 4 | x10000 (0,000) |  |
| Green | 5 | 5 |  |  |
| Blue | 6 | 6 |  |  |
| Violet | 7 | 7 |  |  |
| Grey | 8 | 8 |  |  |
| White | 9 | 9 |  |  |

Not: If no Band 4 is used, tolerance is +/-20%

## Fig 3.4.2 Five Band Military Standard Inductor Colour Code

| Band | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Meaning (See Notes) | Mil. Spec. | Digit or Dec. point | Digit or Dec. point | Digit (or Multiplier) | Tolerance % |
| Gold |  | Decimal point | Decimal point |  | +/-5% |
| Silver | Always Silver double width |  |  |  | +/-10% |
| Black |  | 0 | 0 | 0 (or x 1) | +/-20% |
| Brown |  | 1 | 1 | 1 (or x 10) | +/-1% |
| Red |  | 2 | 2 | 2 (or x 100) | +/-2% |
| Orange |  | 3 | 3 | 3 (or x 1,000) | +/-3% |
| Yellow |  | 4 | 4 | 4 (or x 10,000) | +/-4% |
| Green |  | 5 | 5 | 5 |  |
| Blue |  | 6 | 6 | 6 |  |
| Violet |  | 7 | 7 | 7 |  |
| Grey |  | 8 | 8 | 8 |  |
| White |  | 9 | 9 | 9 |  |

#### Notes:

The military standard for cylindrical inductors specifies 5 coloured bands The same colours are used as in the EIA 4 band code, but:

For band 1, a double width sliver band is used to signify Military Standard.

#### For values less than 10µH:

Bands 2, 3 and 4 indicate the value of inductance in µH

A gold band might be used in either band 2 or band 3. In either of these two bands, gold indicates a decimal point and band 4 is used as a digit instead of a multiplier band.

When no gold band is present in bands 2 or 3, band 4 is a multipier band.

For example:

- If bands 2,3 and 4 are red, gold, red the value would be 2.2 µH

- If bands 2,3 and 4 are gold, yellow, violet the value would be 0.47 µH (470nH)

Band 5 indicates the tolerance between 1% and 20%

#### For values of 10µH or more:

Bands 2 and 3 represent basic value, and band 4 gives the number of zeros.

For example:

If bands 2, 3 and 4 are red, violet, orange the value would be 27000 µH

### Surface Mount Device(SMD) or Chip Inductor Codes.

For inductors of a very small physical size, coloured dots may be used instead of bands. In such cases, The silver dot indicating a Military (Mil) specification will be larger than the other dots and will be placed a the beginning of the dot sequence.

In some cases only a single coloured dot is used,and for their meaning it is neccesary to refer to individual manufacturers data for accurate interpretation.

Dot code examples from Coilcraft Inc. http://www.coilcraft.com/colrcode.cfm

Dot code examples from Viking Tech Corporation. http://www.vikingamerica.com/ftp/NL.pdf

© 2007−2013 Eric Coates MA BSc. (Hons) All rights reserved.