SAMSUNG, (monoblok) CHASSIS: SCV11A, B Model TVP3350, TVP5050, TVP5350 postup - Stand-By P.Std Menu Sleep Power On . Po vstupu do serv.režimu se na obr. objeví menu: Adjustment, Option byte, Reset.

Funkce  Adjustment: AGC - Auto Gain Control, SBT - Sub Bright, SCT - Sub Contrast, SCR - Sub Color, STT - Sub Tint, PDL - Pal Delay, NDL - NTSC Delay, RC - Red Cutoff, GC - Green, Cutoff, BC - Blue Cutoff, RG - Red-Green drive gain, BG - Blue-Green drive gain, PSL - PAL vertical SLope, PVS - PAL Vertical Shift, PVA - PAL Vertical Amplitude, PHS - PAL Horizontal Shift, NSL - NTSC vertical SLope, NVS - NTSC Vertical Shift, NVA - NTSC Vertical Amplitude, NHS - NTSC Horizontal Shift.

OPTION

BIT	ITEM	0	1

7	TTX	NO TTX	TTX

6	TTX SYSTEM	LIST FIRST	FLOF FIRST

5	CONTRAST	90	100

4	TUNER QUANTTITY	1	2

3	TUNER KINDS	2889	0889

2	SYSTEM (L)

1	SYSTEM (D/K)

0	SYSTEM (I)

OPTION 2

BIT	ITEM	0	1

7	NOT USED

6	NOT USED

5	POWER ON AFT	ONCE	TWICE

4	PDC	NO	YES

3	VPS	NO	YES

2	3.58 X-TAL	NO	YES

1	VIDEO PLUS	NO	YES

0	SHOWVIEW	NO	YES

VCR OPTION

BIT	ITEM	0	1

7	NOT USED

6	NOT USED

5	NOT USED

4	HEAD	2-HEAD	4-HEAD

3	SP/LP	SP-ONLY	SP/LP

2	SECAM	N/A	POSSIBLE

1	MESECAM	N/A	POSSIBLE

0	NTSC3.58	N/A	POSSIBLE

SAMSUNG CK5038 ZR\TBWCX CHASSIS SCT11B : POSTUP:  stisk tlač. z DO  : STAND-BY --> P.STD --> HELP --> SLEEP --> POWER ON. Východ z modu - tl. STATUS. Pod krytem DO je skryté tlačítko  HIDDEN , KTERÝM SE OTEVÍRÁ A ZAVÍRÁ SERV. MOD .

SAMSUNG CS7272 PTR\BWX CHASSIS SCT 51A : POSTUP: PICTURE OFF --> SLEEP --> P.STD --> MUTE --> PICTURE ON.

SAMSUNG CK6202 CK7202 CHASSIS SCT12B: POSTUP: STAND-BY --> P.STD --> HELP --> SLEEP --> POWER ON. Východ z menu  HIDDEN nebo STATUS.

SAMSUNG CS-721, CS-723 CHASSIS SCT-52A POSTUP:  PICTURE OFF --> SLEEP --> P.STD --> MUTE --> PICTURE ON. ZMENY A NASTAVENÍ :  CHANNEL A VOLUME.

SAMSUNG CK 6271 WP POSTUP:  1. STISKNOUT  (HIDDEN) - KTERÉ JE SKRYTO POD KRYTEM DO  2. VELMI RYCHLE ZADAT : STAND-BY --> P.STD --> HELP --> SLEEP --> POWER ON.

SAMSUNG CK331EZR48, CK333EZR4X, CK3366ZR4X/BWT, CK3373ZR4X/BWT, CK501EZR4X, CK503EZR4X/BWT, CK5066ZR4X/BWT, CK-5039ZR4X/NWT SHASSIS SCT-13. POSTUP : Z DO ZADAT  SLEEP --> FACTORY --> STAND-BY --> P.STD --> HELP --> SLEEP --> POWER ON.

Objeví se  SERVICE (FACTORY) - servisní režim obsahuje 4 submenu 
Adjustment (Nastavení) vstup do tohoto podmenu provedeme tl.  VOL + nebo VOL -.Výběr položek -  CH- nebo CH+.
Test Pattern  V tomto režimu nastavujeme konvergence obrazovky.
Option bytes (základní nastavení) Bez znalostí změny hodnot v tomto submenu může silně ovlivnit funkci televizoru. Bohožel všechny údaje na obrazovce jsou ve vietnamštine, proto bez specialních tabulek se nedoporučuje nic menit.

Chassis Z68 & RM131

15-10. Manual Alignment Mode (Factory Mode) ? This mode enables manual alignment of the TV set by using the normal user remote control. Service mode is activated by pressing the "hidden service" key on the local keyboard or a sequence of remote control before the stand-by is off. The sequence is as follows : Video prefer - A/V Key - Sleep timer - Power ? The character "FACTORY" is displayed on the left top of the screen while this mode is activated. Factory mode has three sub modes : Main ajust mode, Luminance delay control mode, and White balance control mode. Factory mode is cancelled by pressing the "hidden" key again or "Power Off"

SAMSUNG CS7272 PTR\BWX CHASSIS SCT 51A : POSTUP  : PICTURE OFF --> SLEEP --> P.STD --> MUTE --> PICTURE ON.