PANASONIC  Z7  hibák

Panasonic Z7 Dead except from whistle from psu No line drive - D861 start up supply for IC601 o/c Panasonic Z7 Flashing LED at sw on & no start up Replace o/c R811 (220k 0.5w) in power supply. Panasonic Z7 No sound - OK with sound I/P via scart FM sound I/p ok at pin 2 IC601 M52778SP-A but no o/p at pin 47 - chip duff Panasonic Z7 Screams at sw on, relay operates trip & LED just blinks D861 o/c in psu (8v feed to pin 9 IC601) Cct Diag mistake - anode D861 connects to pin 1 IC851 NOT 852 Panasonic Z7 Service mode access Select chan 60, set sharpness to min, press timer on remote at the same time down arrow on front of set. press str after each adj to store Panasonic Z7 TC14S3R Self check proc Press F button & Vol - button on set & status on r/c at same time. When complete sw off set then on again & retune Panasonic Z7 TC14S3R Weak neg pic - as tho in wrong system EEPROM IC1205 pt no XL24D02P-AA1 corrupted - needs self checking once fitted Panasonic Z7 TC14S3R/T stuck in stby MICRO IC1201 IN PROTECT DUE TO NO +2.6V ON PIN31, R1218 620K 0.5W O/C Panasonic Z7 TC21S3R switched to 16:9 mode CORRUPT EEPROM (NO OPTION CODE) , REPLACE MICRO IC1201 AND EAROM IC1205, INITIALISE BY PRESSING STATUS KEY ON REMOTE, FOLLOWED BY 'V' BUTTON ON TV Panasonic Z7 TX14S3 blank raster, sound OK R560 300K O/C Panasonic Z7 TX14S3 dead, stby LED is low 5V STBY RAIL ONLY 2V, D1202 O/C (1N4150T77) Panasonic Z7 TX14S3 distorted pic and sound, appears to have no sync EPROM IC1205 (XL24D02PAA1) Panasonic Z7 TX14S3T Wont find chans in chan search, those already tuned in are off tune & dist. Tall, flat & greyscale off in AV mode eeprom Part number XL24CO20-AA cured the faults. Caused by dry joint on LOPTx earth tag corrupting EEPROM. Go into"self check" mode. Done by pressing Status button on the remote & V down on set. Shows OPT O2 on screen.Can't store PP until done Panasonic Z7 TX21JT Reverts to st/by after a couple of hrs & keeps cutting out when sw back on EEPROM vots had dropped to 2.3v (5v) due to D1202 going o/c - check other diodes in EEPROM supply line also Panasonic Z7 TX21S3 stuck in stby NO 2.6V TO PIN 31 IC1201(Micro), R1218 O/C FUSIBLE 560K 0.5W PART NO. ERO50PKF5603 Panasonic Z7 TX21S3T goes to stby after 5-10 mins D1208 MA170 LEAKY Panasonic Z7 TX21S3T int. startup, int. blank raster, int. no remote operation LOW SDA/SCL LINES, D307 BZY88C4V7 LEAKY Panasonic Z7 TX21S3T negative pic with AGC overload via tuner (OK via SCART) FAULTY EPROM. BEFORE REPLACING ENTER SERVICE MODE AND WRITE DOWN EPROM VALUES.CHANGE IC AND ENTER ORIG.VALUES, YOU MUST ENTER "SHIPPING CONDITION AUTOTUNE" Panasonic Z7 TX21S3T stuck in stby EEPROM IC1205 (XL24C02PBAB) FAULTY, CHECK BY BYPASSING THE RELAY