Size(mm); 80 x 80 x 15. Model No.; DF128015_U; DF128015_H; DF128015_M; DF128015_L. Voltage(V); 12; 12; 12; 12. Speed(RPM); 4000; 3000; 2500; 2000. Bearing(1); SPB; SPB; SPB; SPB. Current(Amp); 0.45; 0.28; 0.20; 0.14. Watts(W); 5.40; 3.36; 2.40; 1.68. Air Flow(CFM); 48.60; 37.60; 30.80; 23.10. Static Pressure (mm/H2O); 5.90; 3.60; 2.60; 1.70. Noise(dBA); 40.20; 32.10; 27.30; 23.50. PWM(2); v; v; v; v. (1) Available in One Ball Bearing, Dual Ball Bearing. Request special order for Sleeve Bearing (2) "v" avaliable for PWM function (3) MTBF (Operating temp. 25°C): Sleeve Bearing: 20,000 hours, One Ball Bearing: 35,000 hours, Dual Ball Bearing: 50,000 hours. Items; Descriptions. Label Size; 32mm. Frame; PBT+30%GF UL94V-0 (Black). Impeller; PBT+15%GF UL94V-0 (Black). Weight; 60±5 grams. Operating Temperature; - 10 °C ~ + 65 °C (65 %RH). Storage Temperature; - 30 °C ~ + 70 °C (65 %RH). Vibration Test; Displacement Amplitude: 0.75mm(Equivalent 10G) Frequency Range:10Hz<->55Hz/30SEC. Linear Scanning 120 Cycle Endurance Timer Per Axis: 30Min. Orientation: X,Y,Z.. Drop Test; Motor withstands one free body drop from 30 cm in high onto 10 mm thickness of wooden board for each of the three faces in minimum packing condition. Acoustic Noise; 32.6dBA – Curve (Max32.9dBA) Measuring Condition – Under rated voltage in semi-anechoic chamber equipment sound level meter.. Model Number System (example: DF128025SH-3/4G). D; F; 12; 8025; S; H; -3/4; G. Voltage: D: DC A: AC; Type: F: Fan B: Blower S: Overspec. Fan; Voltage Range: 05: 5 V 12: 12V 24: 24V 48: 48V; Size: L x W x H (unit: mm); Bearing System: S: Sleeve P: One Ball B: Dual Ball Q: Liquid System; Speed: U: Super High H: High M: Midle L: Low E: Super Low; Connector: -3: 3-pin with 3 wires connector -3/4: 3-pin to 4pin wires connector /2.0: 2-pin with 2 wires connector /2.54: 2.54-pin with 2 wires connector /2.5: 2.5-pin with 2 wires connector PWM: 4-pin pwm function with 4 wires connector; RoHS.. download..