29-TV4KS

MULTISTANDARD CHASSIS WITH TXT, NICAM, A2 STEREO

COLOUR TELEVISION RECEIVER

# SERVICE MANUAL

## SPECIFICATION

- SYSTEM                         PAL/SECAM; BG/DK/I/LL’ A2/NICAM STEREO

- VIDEO INPUT                    PAL/SECAM/NTSC4,43

- POWER INPUT                   AC 170-245V(50/60Hz)

- POWER CONSUMPTION          95W

- AERIAL IMPEDANCE             75 OHM UNBALANCED

- TUNER                          VOLTAGE SYNTHESIZER TUNING

- RECEIVING CHANNELS          VHF-L E2-S10

VHF-H E5-S41

UHF E21-E69

- PROGRAMME                    MAX.99 PROGRAM MEMORIES

- PICTURE TUBE                   29” (110°)

- SOUND OUTPUT                 ≥2 x 3W

- AV INPUT                        AV1: FULL SCART1;

AV2: SCART2 & RCA

CAUTION: Before servicing the chassis, read the “Safely Precaution”.

“X -Ray radiation Precaution” and “Product Safety Notice” in this manual.

### X-RAY RADIATION PRECAUTION

- Excessive high voltage can produce potentially hazardous X-RAY RADIATION. To Avoid such hazards the high voltage must be specified limit. The normal value of the high voltage of this receiver is 28.5KV +/-500V under 230V AC power source. The high voltage must not exceed 30KV.

- Each time a receiver requires servicing the high voltage should be checked following the HIGH VOLTAGE CHECK procedure in this manual. It is recommended the reading of the high voltage be recorded as a part of the service record. It is important to use an accurate and reliable high voltage meter.

- The primary source of X -RAY RADIATION in this TV receiver is the picture tube. For continued X-RAY RADIATION protection, the replacement tube must be exactly the same type tube as used in this TV receiver.

- Some parts in this receiver have special safety-related characteristics for X-RAY RADIATION protection. For continued safety, parts replacement should be undertaken only after referring the PRODUCT SAFETY NOTICE below.

SAFETY PRECAUTION

WARNING: Service should not be attempted by anyone unfamiliar with the necessary

Precautions on this receiver.

The following are the necessary precautions to be observed before servicing this chassis.

- Since the power supply circuit of this receiver is directly connected to the AC power line. An isolation transformer should be used during any dynamic service to avoid possible shock hazard.

- Always discharge the picture tube anode to the CRT conductive coating before handling the picture tube. The picture tube is highly evacuated and if broken, glass fragments will be violently expelled. Use shatterproof goggles and keep picture tube away from the unprotected body while handling.

- When replacing a chassis in the cabinet, always be certain that all the protective devices are put back in place, such as: nonmetallic control; knobs, insulating covers, shields, isolation resistor-capacitor, network, etc.

- When replacing parts or circuit boards, disconnect the power cord.

- When replacing a high voltage resistor (metal oxide resistor) on circuit board, keep the resistor APP. 10mm(1/2 in.) away from circuit board.

- Connection wires must be kept away from components with high voltage or high temperature.

- If any fuse in this TV receiver is blown, replace it with the FUSE specified in the chassis parts list.

- The receiver is designed to operate with 230V(50Hz) AC mains.

#### PRODUCT SAFETY NOTICE

Many electrical and mechanical parts in this chassis have special safety-related characteristics are often passed unnoticed by a visual inspection and the X-RAY RADIATON protection afforded by them cannot necessarily be obtained by using replacement components rated for higher voltage. The use of substitute replacement parts that do not have the same safety characteristics as specified in the parts list may create shock, fire, X-RAY RADIATION or other hazards.

#### GENERAL ADJUSTMENT

##### AUTOMATIC DEGAUSSING

An automatic degaussing coil is attached around the picture tube, degaussing the tube properly in about one second after the set is switched on. If the receiver is moved or faced on a different direction, the power must be switched off at least 15 minutes in order that the automatic degaussing circuit operated properly. External degaussing is necessary if the automatic degassing proves ineffective after the set is moved.

###### B+ ADJUSTMENT

CAUTION: To avoid X-ray hazards and result in a nominal display width, B+ voltage must be set in the scale of 128V+/-0.5V.

- Make sure the AC power supply is 230V, 50Hz.

- Switch on the TV receiver, tune in an active channel.

- Measure the voltage between C641 on Main P.C. Board by DC voltmeter.

- Set contrast, brightness, color to maximum.

- Adjust VR631 on Main P. C. Board for B+128V+/-0.5V voltage reading.

###### HIGH VOLTAGE CHECK

CAUTION: There is no high voltage adjustment in this chassis, B+ 128V voltage directly relates to the high voltage. The high voltage does not exceed 33KV under any conditions.

- Connect an accurate high voltage meter to the second anode cap of the picture tube.

- Turn on the receiver, set brightness and contrast to minimum (Zero beam current).

- Make sure the high voltage does not exceed 33KV.

- NO matter whether the luminance, contrast and chrominance controls are set to maximum or minimum, the high voltage must be kept under 33KV.

###### FOCUSING

Receive a TV test pattern signal; adjust focus knob to get optimum picture, a well-defined, sharpest display in the center area of the screen.

ADJUSTMENT AND SERVICING THE CHASSIS

###### SERVICE MODE

To enter the service mode, a special remote control which contains an additional SERVICE key must be used. See the illustration.  Press the ‘SERVICE’ key on remote control, TV will display service menu as following table 1 line by line. To select the item by using the ‘P+/P-’ keys. To adjust the value of selected parameter by using the ‘V+/V-’ keys. To quit the service mode, press the ‘P.P.’ key on remote control when in the service mode.

Table 1

| Parameter | Value |
| --- | --- |
| Red gain | 45 |
| DC red | 063 |
| Green gain | 45 |
| DC green | 063 |
| Blue gain | 45 |
| DC blue | 063 |
| Apr threshold | 15 |
| Logo | 6------------ |
| Red Cutoff | 31 |
| Green Cutoff | 31 |

To enter the table 2, press the ‘OK’ key on the remote control for the first time when in the service mode.

Table 2

Service V3.0

| Parameter | Value | Limits |
| --- | --- | --- |
| Tuner AGC | 22 | 0...63 |
| H POS 50 | 40 | 0...63 |
| VPOS 50 | 8(Unused) | 0...15 |
| VAMP 50 | 47(Unused) | 0...63 |
| VLIN 50 | 31(Unused) | 0...63 |
| H POS 60 | 32(Unused) | 0...63 |
| VPOS 60 | 10(Unused) | 0...15 |
| VAMP 60 | 53(Unused) | 0...63 |
| VLIN 60 | 31(Unused) | 0...63 |
| Bright max | 63 | 0...63 |
| Bright min | 00 | 0...63 |
| Sub tint | 32 | 0...63 |
| VCO coarse | 05 | 0...15 |
| VCO fine | 063 | 0...127 |
| VCO coarse L1 | 11 | 0...15 |
| VCO fine L1 | 032 | 0...127 |

To enter the table 3, press the ‘OK’ key for the second time on the remote control when in the service mode.

Table 3

STV 9306

| Parameter | Value | Limits |
| --- | --- | --- |
| V SAW50 | 58 | 0…63 |
| V SAW60 | 30 | 0…63 |
| V SH | 19 | 0…31 |
| V SC | 07 | 0…15 |
| V CC | 08 | 0…15 |
| EW VDC | 13 | 0…31 |
| EW AMP | 14 | 0…31 |
| EW SHAPE | 16 | 0…31 |
| EW TRAP | 15 | 0…31 |

To enter the table 4, press the ‘OK’ key for the third time on the remote control when in the service mode.

Table 4

Design

| Parameter | Value | Limits |
| --- | --- | --- |
| AGC gain | 00 | 00…03 |
| Option 1 | 178 |  |
| Option 2 | 000 |  |
| Option 3 | 134 | With SECAM LL’ - 142 |
| Option 4 | 027 |  |
| Option 5 | 000 |  |
| ST Ttext | 000 | 00…255 |
| H POS OSD | 001 | 001…255 |
| V POS OSD | 01 | 01…63 |
| H POS TXT | 057 | 001…255 |
| V POS TXT | 04 | 01…63 |
| HOTEL MODE | OFF | ON/OFF |
| Volume Limit | 31 | 00…63 |
| SndSt1 | 070 | 000…255 |
| SndSt2 | 079 | 000…255 |

Following are item description of OPTION in more details.

OPTION1

b7 = Over modulation (0=Disable, 1=Enable, it’s suggested to set it to 0 normally.)

b5 = P/N/S crystals application (0=2 Crystals, 1= 1 Crystals, according the quantity of crystal used for color decoding in chassis.)

b4 = Cutoff Loop (0= OFF, 1=ON, it’s suggested to set it to 1 normally)

b3 = Safety_Reset(0=Active, 1=Non, it’s suggested to set it to 0 normally)

b2 = MONO IN(0 = OFF , 1= ON, Depend on the sound decoding solution used in chassis. )

b1 = Sound Demodulation ( 0 = Intercarrier/MONO, 1 =QSS/NICAM, depend on the sound decoding solution used in chassis.)

b0 = logo display(0 =OFF, 1= ON, it’s suggested to set it to 0 normally.)

OPTION2 (set to 0 by default)

b5 = HALF_CONTRAST(0 = OFF, 1=ON)

b4 = Color 6dB (0 = OFF , 1 = ON )

b3 = APR Feature (0 = ON , 1= OFF )

b2 = Black Stretch (0 = ON , 1= OFF )

b1 = Auto Flesh (0 = ON , 1= OFF )

b0 = Coring ( 0 = ON , 1 = OFF )

OPTION3

b7 =SMART (0 = OFF, 1=ON, it’s suggested to set it to 1 normally)

b5 = AVL (0 = OFF, 1=ON, it’s suggested to set it to 0 normally.)

b4 = PIF over modulation (0 = OFF, 1= ON, it’s suggested to set it to 0 normally.)

b3 = Market_France - SECAM LL’ ( 0 = OFF, 1 = ON, depend on the solution of chassis.)

b2 = Manual/Auto cutoff ( 0 = Manual, 1 = Auto cutoff, depend on the CRT for chassis. )

b1 = Mute pin low/high –to control the mute of speaker( 0 = low, 1 = high, depend on the solution of chassis.)

OPTION4

b6 = WIDE MODE ( 0 = OFF,1=ON, it’s suggested to set it to 0 normally.)

b4 = SCART2 ( 0 = OFF,1=ON, depend on the solution of chassis.)

b3 = RGB ( 0 = OFF, 1= ON, depend on the solution of chassis.)

b2 = SVHS ( 0 = OFF, 1= ON, depend on the solution of chassis.)

b1 = AV2 ( 0 = OFF, 1= ON, depend on the solution of chassis.)

b0 = AV1 ( 0 = OFF, 1= ON, depend on the solution of chassis.)

OPTION5

There are 8 bits used in OPTION 5. They are b0, b1, b2, b3, b4, b5, b6 and b7.

For b0, normally, ST suggests to set it to 0.

For b1, b2, they are used to select teletext languages.

For b3, it’s used to enable AV3 and SVHS. When b3 is set to 0, SVHS is disabled. When b3 is set to 1 in binary, the value is 8 in decimal, AV3 is enabled.

For b4, b5, b6 and b7 of OPTION 5 are also used to set the brightness of background in teletext mode. When b4 b5 b6 and b7 are all set to 0, the brightness of teletext background is the same as brightness of TV background. When b4, b5, b6 and b7 are all set to 1, it’s the maximum brightness of teletext background. It’s the minimum brightness of teletext background when b4 is set to 1, and b5, b6 and b7 are all set to 0. You can set these 4 bits according to your favorite brightness of teletext background.

A.  For 11 OSD languages ST92195C7B1/MXX for east Europe, the setting of the b1 and b2 is listed as below.

1.  If b1 = b2 = 0, then the TEXT languages like “ENGLISH, FRENCH, SWEDISH, CZECH, GERMAN, PORTUGUESE (SPANISH), ITALIAN, RUMANIAN” can be decoded;

2.  If b1=1, b2=0, then the TEXT  languages like “ENGLISH, RUSSIAN (BULGARIAN), HUNGARIAN, TURKISH, GERMAN, PORTUGUESE (SPANISH), ITALIAN, RUMANIAN” can be decoded.

3.  If b1=0, b2=1, then the TEXT languages like “POLISH, RUSSIAN (BULGARIAN), SWEDISH, SLOVAK, GERMAN, SLOVENIAN, LITHUANIAN, RUMANIAN” can be decoded.

4.  If b1 = b2 =1, then the TEXT languages like “POLISH, RUSSIAN (BULGARIAN), SWEDISH, CZECH, GERMAN, CROATIAN, LETTISH, RUMANIAN” can be decoded.

C.  For 12 OSD languages ST92195C7B1/MXX for western Europe, the setting of the b1 and b2 is as below.

1.  If b1 = b2 = 0, then the TEXT languages like “ENGLISH, FRENCH, FINNISH, CZECH, GERMAN, PORTUGUESE (SPANISH), ITALIAN, GREEK” can be decoded;

2.  If b1=1, b2=0, then the TEXT languages like “ENGLISH, FRENCH, DANISH, NORWEGIAN, GERMAN, PORTUGUESE (SPANISH), ITALIAN, GREEK” can be decoded.

3.  If b1=0, b2=1, then the TEXT languages like “POLISH, FRENCH, SWEDISH, CZECH, HOLLAND, SERBIAN, ITALIAN, RUMANIAN” can be decoded.

- If b1 = b2 =1, then the TEXT languages like “POLISH, FRENCH, SWEDISH, CZECH, GERMAN, PORTUGUESE (SPANISH), LETTISH, RUMANIAN” can be decoded.

ST Ttext - only for ST engineer

b0 = enable ST to change the process for adjusting Auto gain.

b1,b2,b3 = select the correct process

ROM_M6_P_valid | OSDEPROM_M6_R_valid | ROM_M6_R_valid | EPROM_M6_R_valid

EPROM_M6_R_valid | ROMLESS_H5_P_valid | ROM_H5_P_valid | EPROM_M6_A_valid

/* note: ROMLESS_M6_R_valid == ROM_M6_R_valid*/

b4 = Color transition for SVHS of STV224XH (0 = OFF, 1 = ON).

b5 = Tuner Drift Correction ( 0 = OFF, 1 = ON. )

HPOS OSD

Adjustment for horizontal position of OSD.

VPOS OSD

Adjustment for vertical position of OSD.

HPOS TXT

Adjustment for horizontal position of teletext.

VPOS TXT

Adjustment for vertical position of teletext.

HOTEL MODE

To set HOTEL MODE ON or OFF. When HOTEL MODE is enabled, the end user can not make any adjustment of items in menu INSTALL, AUDIO and ORGANIZE. Menu INSTALL and AUDIO will disappear automatically.

Volume Limit

To set the maximum volume. It’s only available when HOTEL MODE is enabled.

SndSt 1

b0, b1, b2, b3 of SndSt 1 are used to set the sound output level at pin 26 and pin 27 of STV8216 with RF signal. When b0, b1, b2, b3 of SndSt 1 are all set 0, the minimum sound output level will be got at pin 26 and pin 27 of STV8216 with RF signal. When b0, b1, b2, b3 of SndSt 1 are all set 1, it’s 15 in decimal. The maximum sound output level will be got at pin 26 and pin 27 of STV8216 with RF signal.

b4, b5, b6, b7 of SndSt 1 are used to set the “SURROUND” gain at pin 26 and pin 27 of STV8216 with RF & AV signal. When b4, b5, b6, b7 of SndSt 1 are all set 0, the minimum “SURROUND” gain will be got at pin 26 and pin 27 of STV8216 with RF & AV signal. When b4, b5, b6, b7 of SndSt 1 are all set 1, it’s 240 in decimal. The maximum “SURROUND” gain will be got at pin 26 and pin 27 of STV8216 with RF signal.

SndSt 2

b0, b1, b2, b3 of SndSt 2 are used to set the sound output level at pin 26 and pin 27 of STV8216 with AV signal. When b0, b1, b2, b3 of SndSt 1 are all set 0, the minimum sound output level will be got at pin 26 and pin 27 of STV8216 with AV signal. When b0, b1, b2, b3 of SndSt 1 are all set 1, it’s 15 in decimal. The maximum sound output level will be got at pin 26 and pin 27 of STV8216 with AV signal.

b4, b5, b6, b7 of SndSt 2 are used to set the “EQUALIZER” gain at pin 26 and pin 27 of STV8216 with RF & AV signal. When b4, b5, b6, b7 of SndSt 2 are all set 0, the maximum “EQUALIZER” gain will be got at pin 26 and pin 27 of STV8216 with RF & AV signal. When b4, b5, b6, b7 of SndSt 2 are all set 1, it’s 240 in decimal. The minimum “EQUALIZER” gain will be got at pin 26 and pin 27 of STV8216 with RF & AV signal.

###### A.F.C. ADJUSTMENT

Removing any R.F. signal source and prevent any stray signal source from entering the tuner by shorting the tuner input inner contact to the out screen.

Inject the 38.9 MHZ carrier into the tuner IF output pin (pin 11). Select the item ‘VCO Coarse’ in the service mode (see Table 2). The ‘VCO Status…’will appear at bottom of the screen. Then press the ‘AV’ key on remote control, the CPU will adjust automatically until the ‘VCO Status OK’ appears on the screen. If this can not be achieved, i.e. there is no any pattern on the screen, please adjust the T201 first until a stable pattern appears on the screen. And then press the ‘AV’ key on remote control. The CPU will adjust automatically once again until the ‘VCO Status OK’ appears on the screen.

###### GEOMETRY

To adjust the picture position and size, select the appropriate parameter in the service mode and adjust as necessary. Please, see tables 2 & 3.

SPECIFICATION FOR MODEL:

SUPPLY VOLTAGE                  : AC 230V/50Hz(+/-10%)

AERIAL IMPEDANCE                : TV 75 ohm

LOAD                            : 8 ohm

INPUT ANT.                      : 0db=1uV

| DESCRIPTION |  | UNIT |  | LIMIT |  | TEST |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PICTURE SENSITIVITY VHF | dB u V |  | 51 |  |  |
|  | UHF | dB u V |  | 54 |  |  |
|  | CATV | dB u V |  | 51 |  |  |
| 4 | AFC HOLDING RANGE | KHz |  | +≥750; -≥500 |  |  |
|  | SOUND |  |  |  |  |  |
| 5 | MAX.OUTPUT (1KHz) | mV |  | ≥8000 |  |  |
| 6 | THD OUTPUT10%(1KHz) | mV |  | ≥6500 |  |  |
| 7 | REF.THD(1KHz) MONO/STEREO | % |  | <3/3.5 |  |  |
| 8 | S/N | dB |  | <32 |  |  |
| 9 | MIN.HUM | mV |  | <20 |  |  |
| 10 | RESPONSE FH 7KHz/FL 120Hz | dB |  | -3+/-6 |  |  |
| 13 | AUDIO INPUT | Vrms |  | 0.5+/-0.15 |  |  |
| 14 | AUDIO OUTPUT (10K OHM LOAD) | Vrms |  | 0.5+/-0.15 |  |  |
|  | PICTURE |  |  |  |  |  |
| 15 | SCANNING SIZE VERT. | % |  | 90≤K” ≤96 |  |  |
| 16 | SCANNING SIZE HOR. | % |  | 90≤K” ≤96 |  |  |
| 17 | PATTERN DISTORTION | % |  | ≤3.5 |  |  |
| 18 | LINEARITY VERT. | % |  | <7 |  |  |
| 19 | LINEARITY HOR. | % |  | <7 |  |  |
| 20 | LUMINANCE (50% WHITE PATTERN) MAX | Cd/m2 |  | >100 |  |  |

| DESCRIPTION |  | UNIT |  | LIMIT 15” |  | TEST |
| --- | --- | --- | --- | --- | --- | --- |
| 21 | RESOLUTION HORIZONTAL | LINE |  | PAL≥300 |  |  |
| 22 | VERTICAL | LINE |  | PAL≥350 |  |  |
| 23 | COLOUR SENSITIVTTY | dB u V |  | ≤35 |  |  |
| 24 | VIDEO INPUT | Vp-p |  | 1+/-0.3 |  |  |
| 25 | VIDEO OUTPUT (75 OHM LOAD) | Vp-p |  | 1+/-0.3 |  |  |
| 26 | SAFETY HI-POT TEST |  |  |  |  |  |
| 27 | INPUT VOLTAGE 230VAC AC:3000V 1 MIN 10mA |  |  |  |  |  |
| 28 | CRT HIGH VOLTAGE (AT ZERO BEAM CURRNT) | KV |  | 28-31 |  |  |

Remote control

| 29 | DIRECTIVTY 8m(MIN) | 8m(NOR) | OK |
| --- | --- | --- | --- |
| 30 | DIRECTIVTY 30°FROM LIGHT AXIS 5m(MIN) | 7m(NOR) | OK |