# 1. ELECTRICAL ADJUSTMENTS

1.1. Supply Voltage Adjustment

Connect a digital voltmeter to the cathode of diode D611 at the AV1 mode of the TV and set the screen voltage to the minimum with the screen potentiometer. Adjust the main supply voltage (B+) with P601 potentiometer to the following value (after adjustment, readjust Screen voltage).

| Size | CPT Beko Code | CPT Definition | Voltage |
| --- | --- | --- | --- |
| 25" | 056325-TE1 | CPT TES A59EMZ43X07 | 147V |
|  | 056325-VC4 | CPT VC A59EHJ13X38 | 146V |
| 26" | 056426-VC1 | CPT VC W60ELC011X001 PF | 117V |
|  | 056426-VC2 | CPT VC W60ELC011X101 PF SVM | 117V |
| 28" | 056328-PH10 | CPT PH 28"4:3 A66EAK075X11/L (CEK) | 147V |
|  | 056328-SB4 | CPT SEB A66QEW13X10 | 147V |
|  | 056328-TE1 | CPT TES A66EMZ43X07 | 147V |
|  | 056328-VC9 | CPT VC A66EHJ13X62 (POLONYA) | 145 V |
|  | 056329-SS2 | CPT SS A68QCP993X011 PF DY SCD29333 MACR | 128V |
|  | 056428-GS2 | CPT GS W66QDS757X54 (G.Y.K) | 127V |
|  | 056428-GS3 | CPT GS TINT W66QDS770X59S (GYK) PF SVM | 127V |
|  | 056428-PH13 | CPT PH W66ERF022X013 PF WW (CEK) | 142V |
|  | 056428-PH18 | CPT PH W66ERF122X013 PF CEK(MCB DY+TINT) | 142V |
|  | 056428-PH20 | CPT PH W66ECK011X13/L SF MCB DY CEK | 140V |
|  | 056428-PH3 | CPT PH W66ECK011X13 (S.FLAT) FRANCE | 140V |
|  | 056428-PH6 | CPT PH W66ERF022X013 P.FLAT WW | 142V |
|  | 056428-PH8 | CPT PH W66ECK011X13(S.FLAT)HR-PO3 GUN | 140V |
|  | 056428-SS2 | CPT SS W66QDE993X041 PF (MACAR) | 139V |
|  | 056428-VC1 | CPT VC W66EJU023X015 (S.FLAT) | 140V |
|  | 056428-VC6 | CPT VC W66ELC011X001 (GEN2) P.FLAT | 118V |
|  | 056428-VC8 | CPT VC W66EJU023X215 28 16:9 SF SVM | 135V |
|  | 056528-PH1 | CPT PH 28"4:3 A66EAK075X11-50HZ(HRANICE) | 147V |
| 29" | 056329-MT1 | CPT MTH A68EPD10X20 PF D-COM SVM (ALM.) | 131V |
|  | 056329-OR2 | CPT OR A68QFD290X012 PF (KORE) SVM.LI | 126V |
|  | 056329-OR5 | CPT OR A68KTB359X048(P) 1.3R KORE S.TINT | 142V |
|  | 056329-PH3 | CPT PH A68QCU770X77N P.FLAT (MEXICO) | 140V |
|  | 056329-PH6 | CPT PH A68ERF112X013/M PF T.MIDY WW FRA | 128V |
|  | 056329-SB5 | CPT SEB A68QCP993X001 (GERMANY) PF | 128V |
|  | 056329-SB8 | CPT SEB A68QCP993X011 PF DY(SCD-29333) | 134V |
|  | 056329-SS2 | CPT SS A68QCP993X011 PF DY SCD29333 MACR | 128V |
|  | 056329-TE1 | CPT TES.A68EXZ196X107 P.FLAT SVM | 139V |
|  | 056329-VC0 | CPT VC A68AGA25X105 29" SF 1.3R (MEXICO) | 142V |
|  | 056329-VC13 | CPT VC A68ELA021X001 P.FLAT | 133V |
|  | 056329-VC14 | CPT VC A68AGA25X95 29" SF 1.3R WW (MEX.) | 142V |
|  | 056329-VC15 | CPT VC A68AGA20X105 29 SF 1.3R (CHINA) | 142V |
|  | 056329-VC17 | CPT VC A68ELM021X101 P.FLAT SVM ITALY | 134V |
|  | 056329-VC18 | CPT VC A68ELA021X101 P.FLAT SVM ITALYA | 133V |
|  | 056329-VC19 | CPT VC A68ELA011X101 P.FLAT SVM ITALYA | 133V |
|  | 056329-VC20 | CPT VC A68AGA20X105 29 SF 1.3R(DONGGUAN) | 142V |
|  | 056329-VC9 | CPT VC A68ELA011X001 P.FLAT | 133V |
|  | 056333-HT1 | CPT HT A80LZX70X46(C) SVM (CIN) | 128V |
| 32" | 056432-GS2 | CPT GS W76QDD259X11 (P.FLAT) G.Y.K | 127V |
|  | 056432-PH8 | CPT PH W76ESF011X14 CEK SF P03GUN SVM | 140V |
|  | 056432-PS2 | CPT PS W76EKW10X21 P.FLAT | 140V |
|  | 056432-SS1 | CPT SS W76QEN693X041 P.FLAT (HUNG)SNG. | 140V |
|  | 056432-VC2 | CPT VC W76EGV023X015 (S.FLAT) | 133V |
|  | 056432-VC6 | CPT VC W76ELC011X101 (GEN2) P.FLAT | 135V |
| 33" | 056333-HT1 | CPT HT A80LZX70X46(C) SVM (CIN) | 128V |
|  | 056333-PH4 | CPT PH A80QCF330X22N (KOREA) DY.SIZ | 159V |

2.  SERVICE ADJUSTMENTS

To enter “SERVICE MENU”  , edit “9301”  by remote control while main menu on the secreen.To exit from service menu ,press TV/TXT button.

2.1. AGC Adjustment

- Enter service menu

- Enter “OPTION”  sub menu and set  “AGC METHOD”  item as a  “SYNC+PEAK WHITE”

- Exit from “OPTION”

- Enter “IF ADJ.”  and set “AGC” as “2” and “AGC FOR VHF I” as “2”

Not: For Panasonic DB2G3 (PS4) tuner , set “AGC” as “4” and “AGC FOR VHF I” as “4”

- Set   "PIP AGC" as "15" and "PIP AGC FOR VHF I"  as "20"

- Exit from the service menu

2.2. Screen Adjustment

- Enter the Service Menu and select “VIDEO ADJ. I”  and find “SCREEN ADJ.”

- Enter the value from the table of preset values

- Press “ok” button on RC

- Adjust the screen potentiometer to the level where the screen is just black.

- By pressing “ok” please see the picture on TV

- Exit from the Service menu.

2.3. Geometry Adjustments

- In C7 and C8 chassis ,There is different  geometric adjustment memory for PAL/SECAM and NTSC.  For PAL/SECAM “50Hz GEOM.” , "50Hz GEOM. HOR.I" ve "50Hz GEOM. EHT", For NTSC  "60Hz GEOM.", "60Hz GEOM. HOR.I" ve "60Hz GEOM. EHT" menus are used.For all version both of them have to be adjust.

- Apply a Cross Hatch Test pattern.

- Enter Service Menu

- If the product has “TILT “ propertis, Please select TILT as available under the “SOUND OPTION” and set “TILT 0” as “32”

- Set the value of this item as given in the preset table EHT TRESHOLD" , "EHT TIME CONSTANT" , "VERTICAL EHT 1" , "VERTICAL EHT 2", "HORIZONTAL EHT 1", "HORIZONTAL EHT 2", "EHT DTC", "EHT P1", "EHT P2", "EHT A1", "EHT A2", "START LINE MEASUR.", "VER. BLANKING STOP", "VER. BLANKING START", "HOR. BLANKING STOP" ve "HOR. BLANKING START"

- Adjust  item "HORIZONTAL WIDTH, "VERTICAL AMPLITUDE"  , "VERTICAL SHIFT" to vertical shift , "LINEARITY", "S-CORRECTION", "VERTICAL ANGLE", "VERTICAL BOW", "HORIZONTAL SHIFT" , "TRAPEZE CORR. I" "CUSHION CORR I.", "UPPER CORNER 1 I" and "UPPER CORNER 2 I"  "LOWER CORNER 1 I" and "LOWER CORNER 2 I" , "HORIZONTAL OSD POSITION" "VERTICAL OSD POSITION"

- Adjust 16:9 TV's  4:3 picture format or  4:3 TV's 16:9 picture format by entering  "50HZ GEOM. HOR.I" sub menu.Adjust the items  "TRAPEZE CORR.II", "CUSHION CORR.II" , "UPPER CORNER 1 II"  "UPPER  CORNER 2 II", "LOWER CORNER 1 II" ve "LOWER CORNER 2 II"

- For NTSC geometry, connect NTSC signal to Scart 1(AV1). Enter  "60Hz GEOM. VER.", "60Hz GEOM. HOR.I" and  "60Hz GEOM. EHT" and adjust geometry like PAL/SECAM

2.4. White Balance Adjustment

- Enter the Service Menu, select "VIDEO ADJ. I" menu

- Set  "G. CUTOFF" as  "156" , "G. DRIVE" as "356".  Change "R. DRIVE" high red light, "B. DRIVE" high blue ligt , "R. CUTOFF"  red low light and "B. CUTOFF" blue low ligt .If  white color can not be setted , change the value of  "G. CUTOFF"  and "G. DRIVE" (The values in the preset table  are aproximate and these values have to be setted  for each tv sets)

- Exit from the Service menu.

2.5. Feature Options

| FEATURE OPTIONS AND SERVICE MENU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| *AV1 YUV | AVAILABLE : Scart1 component input available , N/A : Scart1 component input not available |  |  |  |  |  |  |  |  |
| AV2&AV3 | AV2&AV3 N/A : Scart2 and AV3 not available |  |  |  |  |  |  |  |  |
|  | AV2 AVAIL.-AV3 N/A : Scart2 yes, AV3 no |  |  |  |  |  |  |  |  |
|  | AV2&AV3 AVAILABLE : Scart2 and AV3 yes |  |  |  |  |  |  |  |  |
| AV4 | NONE : Front AV no |  |  |  |  |  |  |  |  |
|  | CVBS + SVHS : Front AV + S-VHS available |  |  |  |  |  |  |  |  |
|  | CVBS ONLY :Only Front AV available. (S-VHS not) |  |  |  |  |  |  |  |  |
|  | SVHS ONLY : Only S-VHS available. (Front AV not) |  |  |  |  |  |  |  |  |
| TELETEXT | DEFAULT : Teletext ; FASTEXT : Fastext ; TOPTEXT : Toptext |  |  |  |  |  |  |  |  |
|  | FASTEXT + TOPTEXT : Fastext + Toptext |  |  |  |  |  |  |  |  |
| LANGUAGE | It is used to select menu language |  |  |  |  |  |  |  |  |
|  | A : English-German-French-Italian-Spanish-Portuguese-Greek-Turkish-Dutch-Swedish- |  |  |  |  |  |  |  |  |
|  | Danish-Norwegian-Finnish-Slovenian-Polish-Hungarian-Russian-Hebrew-Romanian |  |  |  |  |  |  |  |  |
|  | Croatian-Czech-Slovak-Albanian-Bulgarian- Macedonian-Serbian |  |  |  |  |  |  |  |  |
|  | B : English-German-French-Italian-Spanish-Portuguese-Greek-Turkish-Dutch-Swedish |  |  |  |  |  |  |  |  |
|  | Danish-Norwegian-Finnish-Slovenian-Polish-Hungarian-Russian-Romanian-Croatian |  |  |  |  |  |  |  |  |
|  | Czech-Slovak-Albanian-Bulgarian- Macedonian-Arabic-Persian |  |  |  |  |  |  |  |  |
| TXT TABLE | AUTO : Automatic selection of TXT table with Menu Language. |  |  |  |  |  |  |  |  |
|  | WEST : English,French,German,Turkish,Spanish,Italian,Finnish,Swedish,Norwegian,Dannish |  |  |  |  |  |  |  |  |
|  | EAST : Polish,French,Hungarian,Czech,German,Slovak,Italian,Romanian |  |  |  |  |  |  |  |  |
|  | GREEC :English,French,Turkish,German,Finnish,Norwegian,Swedish,Danish,Spanish, |  |  |  |  |  |  |  |  |
|  |  | Italian,Greek |  |  |  |  |  |  |  |
|  | CYRILLIC :English,Russian,German,Czech,Estonian,Ukrainian,Letvian, |  |  |  |  |  |  |  |  |
|  | ARABIC :English,French,German,Turkish,Spanish,Italian,Finnish,Swedish,Norwegian, |  |  |  |  |  |  |  |  |
|  |  | Dannish,Arabic |  |  |  |  |  |  |  |
|  | FARSI : English,French,German,Turkish,Spanish,Italian,Finnish,Swedish,Norwegian, |  |  |  |  |  |  |  |  |
|  |  | Dannish,Persian |  |  |  |  |  |  |  |
|  | HEBREW : English,French,German,Turkish,Italian,Finnish,Swedish,Norwegian,Dannish, |  |  |  |  |  |  |  |  |
| MAIN TUNER | To select used main tuner |  |  |  |  |  |  |  |  |
|  | Philips; Panasonic DB2G3; Temic; Panasonic D44G3; Sharp or Alps |  |  |  |  |  |  |  |  |
| *PIP TUNER | To select PIP tuner |  |  |  |  |  |  |  |  |
|  | Philips; Panasonic DB2G3; Temic; Panasonic D44G3; Sharp or Alps |  |  |  |  |  |  |  |  |
| *PIP | AVAILABLE : PIP available ; N/A : PIP not available. |  |  |  |  |  |  |  |  |
| BG | EUROPE : BG Europe ; NEW ZELLAND : BG New Zelland. |  |  |  |  |  |  |  |  |
|  | AUSTRALIA : BG Australia ; N/A : BG Not available. |  |  |  |  |  |  |  |  |
| DK | AVAILABLE : DK available ; N/A : DK not available |  |  |  |  |  |  |  |  |
| I | AVAILABLE :" I" available ; N/A :" I " not available. |  |  |  |  |  |  |  |  |
| L/L' | AVAILABLE : LL' available ; N/A : LL' not available. |  |  |  |  |  |  |  |  |
| *NICAM | AVAILABLE : Nicam available ; N/A : Nicam not available. |  |  |  |  |  |  |  |  |
| *DOLBY VIRTUAL | AVAILABLE : Virtual Dolby available ; N/A : Virtual Dolby not available. |  |  |  |  |  |  |  |  |
| HEADPHONE | AVAILABLE : Headphone available ; N/A : Headphone not available. |  |  |  |  |  |  |  |  |
| *BASS OPTION | DYNAMIC BASS : Dinamik Bass open ; SUBWOOFER : Subwoofer open. |  |  |  |  |  |  |  |  |
|  | NONE : Bass option not available. |  |  |  |  |  |  |  |  |
| CARRIER MUTE | VIA MSP : For all market except Turkey; VIA MICRO : For Turkey market. |  |  |  |  |  |  |  |  |
| *FM RADIO | NONE: Radio and EPG closed. FM RADIO: Only radio open. |  |  |  |  |  |  |  |  |
| / TV GUIDE | TV GUIDE : Only EPG open. |  |  |  |  |  |  |  |  |
| *LTI & COMB | ON: LTI ve Comb. Filter opened ; OFF: LTI ve Comb. Filter closed |  |  |  |  |  |  |  |  |
| NOT: | LTI&COMB selected for C7 chassis it is OFF, for C8 chassis it is ON |  |  |  |  |  |  |  |  |
| *CTI | ON : Color Transient Improvement properties opened |  |  |  |  |  |  |  |  |
|  | OFF :Color Transient Improvement properties closed. |  |  |  |  |  |  |  |  |
|  | MENU :Color Transient Improvement properties can be selected on user menu. |  |  |  |  |  |  |  |  |
| NOT: | CTI normaly selected for chassis C7 as OFF and for chasis C8 as MENU |  |  |  |  |  |  |  |  |
| *PAT | AVAILABLE : Picture and Text available |  |  |  |  |  |  |  |  |
|  | N/A : Picture and Text not available |  |  |  |  |  |  |  |  |
| SIMPLE HOTEL | AVAILABLE : Hotel TV , N/A : Normal TV |  |  |  |  |  |  |  |  |
| HOTEL MAX VOL | Hotel TV maximum volume level can be selected |  |  |  |  |  |  |  |  |
| CRT | 4:3 : 4:3 TV's ; 16:9 : 16:9 TV's. |  |  |  |  |  |  |  |  |
| *SVM&DCI | ON : Scan Velocity Modulation available ; OFF : Scan Velocity Modulation not available. |  |  |  |  |  |  |  |  |
|  | MENU : SVM option can be selected via user menu |  |  |  |  |  |  |  |  |
| BLUEBACK | ON : Blueback opened ; OFF : Blueback closed |  |  |  |  |  |  |  |  |
| NOT: | Normaly BLUEBACK is selected for C7 as OFF and for C8 as ON |  |  |  |  |  |  |  |  |
| *CALCULATOR | AVAILABLE : Caculater opened ; N/A : closed |  |  |  |  |  |  |  |  |
| *GAME | AVAILABLE : Game opened ; N/A : closed. |  |  |  |  |  |  |  |  |
| *ALARM TIMER | AVAILABLE : Alarm opened ; N/A : Alarm closed |  |  |  |  |  |  |  |  |
| PROTECTION | N/A : Protection circuitry not available ; |  |  |  |  |  |  |  |  |
|  | BCL ONLY : Only Beam Current Limitation protection available. |  |  |  |  |  |  |  |  |
|  | VERTICAL ONLY : Only vertical protection available. |  |  |  |  |  |  |  |  |
|  | BCL & VERTICAL : Both of them available. |  |  |  |  |  |  |  |  |
| NOT: | All C7-C8 protection default value is N/A . |  |  |  |  |  |  |  |  |
|  | If the related circuitry does not exist ,DO NOT SELECT AVAILABLE |  |  |  |  |  |  |  |  |
| *PANORAMA | AVAILABLE : Panorama mode opened ; N/A : Panorama mode closed. |  |  |  |  |  |  |  |  |
|  | NOT1: This item not functioanal for 4:3 TV's |  |  |  |  |  |  |  |  |
| *DYNAMIC FOCUS | ON : Dynamic Focus available ; OFF : Dynamic Focus not available. |  |  |  |  |  |  |  |  |
| KEYBOARD | P- P+ V- V+ ; Selects this button configuration on TV. |  |  |  |  |  |  |  |  |
|  | MENU P/V - + ; Selects this button configuration on TV |  |  |  |  |  |  |  |  |
| NOT: | KEYBOARD items have to be selected with product properties |  |  |  |  |  |  |  |  |
| DEMO | 0 : Automatic tuning at first opening is closed |  |  |  |  |  |  |  |  |
|  | 1 :Automatic tuning at first opening is opened. |  |  |  |  |  |  |  |  |
| *TILT | N / A : Tilt adjutment not available ; Available :Available |  |  |  |  |  |  |  |  |
| AGC METHOD | Microcontroller AGC adjustment method. Sync peak white ; Fixed. |  |  |  |  |  |  |  |  |
| CRT4:3,MODE16:9 | VIA FLYBL : Cutoff lines make not visable by moving top. |  |  |  |  |  |  |  |  |
|  | DISABLE CUTOFF : Cutoff lines directly disabled |  |  |  |  |  |  |  |  |
| NOT: | Default vaule is VIA FLYBL |  |  |  |  |  |  |  |  |
| NOT: | * items not availabe in C7 Mono S/W. |  |  |  |  |  |  |  |  |

  - Factory Settings for Service Mode (Preset Table)

- Values given in Preset Table are typical values and can vary according to the CRT type.

- Also in the list some changing value are givens acording to CPT type.CPT  called in the list as last  digits after “-“ of CPT Beko codes.CPT Beko codes are given in Voltage adjustment section.

- PF means Pure Flat  and SF means Super Flat.

- The items marked with  “ *  “ in Preset Table  can be changed for each tv set.