| Philips color tv chassis L6.1 Tips |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Goodmans |  | 1465TS L6.1 | Dead - psu blown due to EHT lead arcing in LOPTx |  | Fit PSU kit 482231010663 & LOPTx 482214010562 & LOPTr |  | plamensl | 14-09-2004 |
| Goodmans |  | 1465TS L6.1 | Weak contrast - black vert lines |  | C2427 33nF 250v in beam lim |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | After a while white screen. |  | Check/replace LOT L5422. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Bad focus or focus varies in a new set or after CRT replacement |  | Possible solution: Short circuit the focus and the G2 pins of the picture tube with the aquadag coating. Switch the set on for approximately 5 seconds. After this return the set to its original condition. Check if the focus adjusted normally. If not repla |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Bending picture depending on its white contents. |  | Change R3420 (see diagram A1) from 12k to 18K (service code 4822 116 52251) IRIS CODE : 1353 |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Code-number supply-repair kit L6.1 |  | Code-number supply-repair kit L6.1 ---------------------------------- In the service manual of the L6.1 the codenumber of the supply-repair kit is missing. The code-number is 4822 310 10663. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Colour flashes appears or loss of chroma in SECAM standard |  | Change R3433 (see Diagram A1) from 39K to 56K service code 4822 051 20563 |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Cracks in sound during fast changing of picture from black to white. |  | Change C2425 to 2200uF 25V.codenumber 4822 124 42449. REMARKS : This only occurs if contrast is at maximum. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Cracks in sound during fast changing of picture from black to white. |  | New solution: Replace 9001 by 39 Ohm NFR = 4822 052 10399 and add 470 uF between 9001/3013 and ground (470 uF=4822 124 80791) |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Cross picture (You can partly see the picture of the next station) |  | Add new Saw filter (CE marking module = 4822 212 11352). This filter can be switched between BG and DK. How to adapt the set: * remove saw filter on position 1001 and replace by CE marking module. * solder the black wire in the hole MCE1 near .P |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Dead - chopper s/c, mains fuse o/c etc |  | Fit repair kit - still dead with squealing from tfmr - pin hole in BUT11AX encapsulation (measured ok out of cct) |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | dead, LOPTr s/c |  | replace C2503 with 33nF 250V (532212142489), R3512 with 68k, D6510 with 12V zener |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Does not find all programs during search (for TXT and NO-TXT versions) |  | New software will solve this problem. Change software into new version;codenumbers are still identical 4822 209 14646:TXT 1 p Mono Pan-European L6TXTMOEU-1.1 3/4 keys:SAA5290ZP/072 4822 209 15425:TXT 1 p Mono Greek/Turkisch L6TMOGT-1.1 |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Does not find all programs during search. |  | Replace microprocessor IC7600 with version L6NOTXT-2.0, codenumber 4822 209 13085. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Focus not optimal |  | Check/replace LOT L5422. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Hotel mode |  | Hotel mode ---------- Only for L6.1 chassis with 4 buttons To activate this mode do the following: - select channel 38 - push vol- and vol + on the TV set AND OSD on the remote control simultaneously. - you now enter the hotel mode. To leave the hotel |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Hotel mode |  | Hotel mode ---------- Only for L6.1 with 3 buttons To activate/de-activate: - select channel 38 - push OSD menu and select Hotel Mode |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Hotel mode, hospital1 mode, hospital2 mode |  | Hotel mode, hospital1 mode, hospital2 mode ------------------------------------------ L6.1 chassis can be put in hotel-mode, hospital1 or hospital2 mode. When the set is in a hospital mode, the set can leave this mode by pushing the PIP on-off or the PIP |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Inoperative (squeaking sound) |  | Cause is defective LOT: 4822 140 10562 Probably the power supply is also defective: 4822 310 10663 Supply-kit. Supply and EHT can be tested seperately by opening jumper 9420. |  | plamensl | 14-09-2004 |
| Philips |  | L6.1 | Lock up & lose pic int |  | Mod reqd- replace Tr7505 BC548B with BC337-40; R3500 from 330R to 470R; if fault persists change EEPROM chip pt no 4822 2096 2098 |  | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No colour or colour flashes |  | Change R3420 (see diagram A1) from 12k to 18K (service code 4822 116 52251) | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No contrast |  | Check/replace R3421 and R3420 (15K). | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No function, but supply and LED are on |  | Check contrast and brightness | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No picture no sound, only red LED |  | Check for brooken tracks near SCART connector and LOT. Check also R3430 near lot: 14" 1k 4822 052 11102 20" 820R 4822 052 10821 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No picture, no sound. Only red led on. |  | Check/replace diode D6501 (5V6). | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No picture, only sound. |  | Check/replace T7402, T7400, R3404, R3405 and C2404. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No picture. Red LED is on. Hum from speakers. +96S too low. |  | Check/replace LOT L5422. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No sound |  | Check and replace if necessary R3523 (3E3, 0.3W) from pt 6 SMPS transformer. If interrupted, +8SB (9V4) on audio amplifier (pt 1) is not present. Resistor 3E3, 0.3W is replaced by 0.5 W: 4822 052 11338. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | No sound in mono 1 Watt versions. |  | Change R3523 to 3E3 0.5W (4822 052 11338) | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Noise in the sound at program switching |  | Check and replace if necessary item 7700 by TDA7052A with codenumber 4822 209 33763. Remove item R3139 and R3702. Add item R3169 with codenumber 4822 051 20103 (10K) and add item R3615 with codenumber 4822 051 20562 (5K6). | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Noise on one channel, wrong stereo indication |  | Noise on one channel, or stereo indication while transmission is mono (or with NICAM reception). On audio module, change IC7353 to MSP3400C-C6 (4822 209 13099) | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | ONLY FOR 20" CRT A48ECR141X18 changed into A48ECR |  | ONLY FOR 20" CRT A48ECR141X18 changed into A48ECR143X18 ------------------------------------------ For service code 4822 131 11314 was initially delivered picture tube A48ECR141X18. This picture tube is however no longer available and is replaced by type | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Picture jumps to the left, narrow picture |  | Change IC7100 TDA8361 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Picture size 20" too big |  | There is no adjustment for H-width nor for the power supply. When R3507 is changed, the output voltage (+96S) will change. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Servicing the Philips L6.1 Chassis |  | See TV Mag Jan 2000 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Set does not always start-up |  | Check and replace if necesarry item 3134 by a 27 Ohm resistor with service-code 4822 053 10279 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Set total inoperative, no LED |  | Check/replace T7504, D6506/6514, F1501, R3514/R3515 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Set totally inoperative, no LED |  | Check/replace first R3508 (6K8), then mount supply kit if necessary. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Set totally inoperative. No LED on. |  | Check/replace LOT L5422 and line transistor T7422 (BUT11AX). | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Sometimes hang-up of the set; only for teletext version ! |  | In L6.1 chassis change pos 7505 to BC337-40, codenumber 4822 130 41344 and change pos 3500 to 470 ohm, service codenumber 4822 051 20471. (All changes are in diagram A1, power supply.) REMARKS : Only teletext version. | plamensl |  |
| Philips | L6.1 |  |  | Strange crack/noise if the sets are broadcasted by cable network |  | Apply new IF saw filter called CE (Marking) module (4822 212 11352). This module can be switched between BG and DK. How to adapt the set: * first remove saw filter on pos 1001 and replace this by CE module 4822 212 11352. * Solder the black wire i | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Supply hiccupping. |  | Check/replace line transistor T7422 (BUT11AX). | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Supply improvement |  | Supply improvement ------------------ If line transistor has short-circuit, then also the supply can become defective. To prevent this, mofify supply: 2503 33nf 5322 121 42489 3512 68K 4822 116 52297 6510 BZX79-C12 4822 130 34197 Check also whether C2512 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Supply inoperative. |  | Check/replace R3518 120KOhm. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Timer does not work properly above 4 hours. |  | Change software into new version;codenumbers are still identical. 4822 209 14646:TXT 1 p Mono Pan-European L6TXTMOEU-1.1 3/4 keys:SAA5290ZP/072 4822 209 15425:TXT 1 p Mono Greek/Turkisch L6TMOGT-1.1 3/4 keys:SAA5290ZP/074 482 | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | Violet bar at the right side of the picture with Canal+ decoder. |  | Add on pos C2423 (Position unbekannt) a capacitor of 680 pF. Codenumber 4822 126 12269. | plamensl | 14-09-2004 |
| Philips | L6.1 |  |  | White picture with flyback lines; VG2 reacts at focus adjust. |  | Repair picture-tube by artificial flashover | plamensl | 14-09-2004 |
| Philips | L6.1 14PT1332/05 |  |  | Field collapses to about 1" aft warm up |  | Scan coils - measured 12.6R cold & 2R2 when heated | plamensl | 14-09-2004 |
| Philips | L6.1 14PT1532 |  |  | No picture, hissing noise from speakers ( like no aerial), front LED not lit. Only 1v on micro supply line |  | Replaced 7505 BC548B, 5V reg from 5500 transformer. Philips Mod. Replace BC548B with a BC337-40 & change R3500 from 330R to 470R | plamensl | 14-09-2004 |
| Philips | L6.1 20PT1552 |  |  | Contrast poor |  | TDA8361 | plamensl | 14-09-2004 |
| Philips | L6.1 21PT1532 |  |  | Int locks up or loses pic |  | Carry out the following mod Remove Tr7505 (BC548B) & fit a BC337-40, change R3500 from 330ohm to 470ohm (these two are in the 5v rail) if any unusual faults persist then suspect corrupted EEPROM (part No 4822 2096 2098) | plamensl | 14-09-2004 |
| Philips | L6.1/6.2 |  |  | For repairing PSU |  | Mod kit available from CHS P10663 ?8.34 | plamensl | 14-09-2004 |
| Philips | L6.1AA |  |  | off - tr 7503 s/c R3514/3515 o/c |  | fit psu kit & check lop tr, lopt, opto coupler & D6426 NOTE- will not run on dummy load - use variac with lop tr removed | plamensl | 14-09-2004 |
| Philips | L6.1AA |  |  | Sound goes off & no R/C functs when heated |  | Micro faulty | plamensl | 14-09-2004 |
| Pye | L6.1AA M2052/05 |  |  | Dead - off with a bang |  | Replace suspect LOPTx, s/c cap & s/c LOPTr 5422 LOPTx 20" L6 (4822 140 10561) 2422 560nF 5% 250V (4822 121 42634) 7422 BUT11APX (4822 130 10206 | plamensl |  |

L6.1

Mindegyik tápfeszültség csak fele a névlegesnek.

C2501 (1,5nF/2KV) megszakadt.