PHILIPS ES1 sasszi

Vg2 Adjustment

- In the frame-blanking period of the R, G, and B signals applied

- to the CRT, the video processor inserts a measuring pulse with

- different DC levels. Measure the black level pulse during the

- vertical flyback at the RGB cathodes of the CRT.

- 1. Connect the RF output of a pattern generator to the

- antenna input. Input a "black" picture (blank screen on CRT

- without any OSD info) test pattern.

- 2. In the SAM mode, set the “Normal Red”, “Normal Green”

- and “Normal Blue” values to “0” for “White Tone”.

- 3. Disable the black current loop (via the AKB bit).

- 4. Use the MENU key to enter the "user" menu, select

- "Picture", and set "Brightness" and "Contrast" to “0”.

- 5. Set the oscilloscope to 20 V/div and the time base to 20 us/

- div. Use external triggering on the vertical pulse.

- Caution: use a trigger point on the "cold" side!

- 6. Ground the scope on the CRT panel ("cold" side) and

- connect a 10:1 probe to one of the cathodes of the picture

- tube socket (see circuit diagram B1).

- 7. Measure at test points F338, F339 and F341 on the picture

- tube socket the DC-level of the measuring pulse (1st full

- line after the frame blanking) with respect to earth.

- 8. Select the pin with the highest level found and adjust

- V_cutoff by means of the Vg2-potmeter (lowest-one) on the

- Line Output Transformer (LOT) to 160 +/- 5 VDC (for all

- screen sizes).

- 9. Reset "Contrast" and "Brightness" to their original values.