# BEKO C7/C8 Usys értékek

1.1. Supply Voltage Adjustment

Connect a digital voltmeter to the cathode of diode D611 at the AV1 mode of the TV and set the screen voltage to the minimum with the screen potentiometer. Adjust the main supply voltage (B+) with P601 potentiometer to the following value (after adjustment, readjust Screen voltage).

| Size | CPT Beko Code | CPT Definition | Voltage |
| --- | --- | --- | --- |
| 25" | 056325-TE1 | CPT TES A59EMZ43X07 | 147V |
|  | 056325-VC4 | CPT VC A59EHJ13X38 | 146V |
| 26" | 056426-VC1 | CPT VC W60ELC011X001 PF | 117V |
|  | 056426-VC2 | CPT VC W60ELC011X101 PF SVM | 117V |
| 28" | 056328-PH10 | CPT PH 28"4:3 A66EAK075X11/L (CEK) | 147V |
|  | 056328-SB4 | CPT SEB A66QEW13X10 | 147V |
|  | 056328-TE1 | CPT TES A66EMZ43X07 | 147V |
|  | 056328-VC9 | CPT VC A66EHJ13X62 (POLONYA) | 145 V |
|  | 056329-SS2 | CPT SS A68QCP993X011 PF DY SCD29333 MACR | 128V |
|  | 056428-GS2 | CPT GS W66QDS757X54 (G.Y.K) | 127V |
|  | 056428-GS3 | CPT GS TINT W66QDS770X59S (GYK) PF SVM | 127V |
|  | 056428-PH13 | CPT PH W66ERF022X013 PF WW (CEK) | 142V |
|  | 056428-PH18 | CPT PH W66ERF122X013 PF CEK(MCB DY+TINT) | 142V |
|  | 056428-PH20 | CPT PH W66ECK011X13/L SF MCB DY CEK | 140V |
|  | 056428-PH3 | CPT PH W66ECK011X13 (S.FLAT) FRANCE | 140V |
|  | 056428-PH6 | CPT PH W66ERF022X013 P.FLAT WW | 142V |
|  | 056428-PH8 | CPT PH W66ECK011X13(S.FLAT)HR-PO3 GUN | 140V |
|  | 056428-SS2 | CPT SS W66QDE993X041 PF (MACAR) | 139V |
|  | 056428-VC1 | CPT VC W66EJU023X015 (S.FLAT) | 140V |
|  | 056428-VC6 | CPT VC W66ELC011X001 (GEN2) P.FLAT | 118V |
|  | 056428-VC8 | CPT VC W66EJU023X215 28 16:9 SF SVM | 135V |
|  | 056528-PH1 | CPT PH 28"4:3 A66EAK075X11-50HZ(HRANICE) | 147V |
| 29" | 056329-MT1 | CPT MTH A68EPD10X20 PF D-COM SVM (ALM.) | 131V |
|  | 056329-OR2 | CPT OR A68QFD290X012 PF (KORE) SVM.LI | 126V |
|  | 056329-OR5 | CPT OR A68KTB359X048(P) 1.3R KORE S.TINT | 142V |
|  | 056329-PH3 | CPT PH A68QCU770X77N P.FLAT (MEXICO) | 140V |
|  | 056329-PH6 | CPT PH A68ERF112X013/M PF T.MIDY WW FRA | 128V |
|  | 056329-SB5 | CPT SEB A68QCP993X001 (GERMANY) PF | 128V |
|  | 056329-SB8 | CPT SEB A68QCP993X011 PF DY(SCD-29333) | 134V |
|  | 056329-SS2 | CPT SS A68QCP993X011 PF DY SCD29333 MACR | 128V |
|  | 056329-TE1 | CPT TES.A68EXZ196X107 P.FLAT SVM | 139V |
|  | 056329-VC0 | CPT VC A68AGA25X105 29" SF 1.3R (MEXICO) | 142V |
|  | 056329-VC13 | CPT VC A68ELA021X001 P.FLAT | 133V |
|  | 056329-VC14 | CPT VC A68AGA25X95 29" SF 1.3R WW (MEX.) | 142V |
|  | 056329-VC15 | CPT VC A68AGA20X105 29 SF 1.3R (CHINA) | 142V |
|  | 056329-VC17 | CPT VC A68ELM021X101 P.FLAT SVM ITALY | 134V |
|  | 056329-VC18 | CPT VC A68ELA021X101 P.FLAT SVM ITALYA | 133V |
|  | 056329-VC19 | CPT VC A68ELA011X101 P.FLAT SVM ITALYA | 133V |
|  | 056329-VC20 | CPT VC A68AGA20X105 29 SF 1.3R(DONGGUAN) | 142V |
|  | 056329-VC9 | CPT VC A68ELA011X001 P.FLAT | 133V |
|  | 056333-HT1 | CPT HT A80LZX70X46(C) SVM (CIN) | 128V |
| 32" | 056432-GS2 | CPT GS W76QDD259X11 (P.FLAT) G.Y.K | 127V |
|  | 056432-PH8 | CPT PH W76ESF011X14 CEK SF P03GUN SVM | 140V |
|  | 056432-PS2 | CPT PS W76EKW10X21 P.FLAT | 140V |
|  | 056432-SS1 | CPT SS W76QEN693X041 P.FLAT (HUNG)SNG. | 140V |
|  | 056432-VC2 | CPT VC W76EGV023X015 (S.FLAT) | 133V |
|  | 056432-VC6 | CPT VC W76ELC011X101 (GEN2) P.FLAT | 135V |
| 33" | 056333-HT1 | CPT HT A80LZX70X46(C) SVM (CIN) | 128V |
|  | 056333-PH4 | CPT PH A80QCF330X22N (KOREA) DY.SIZ | 159V |