Tantalum Capacitor Codes.

How to read a Tantalum capacitor with Numeric coding (Sample 10µF, 25V).

How to read a Tantalum capacitor with Color coding (Sample 6.8µF, 6.3V).

Tantalum Color Code Table.

| Color | 1st Digit | 2nd Digit | Multiplier Dot | Voltage |
| --- | --- | --- | --- | --- |
| Black | 0 | 0 | x 1 | 10V |
| Brown | 1 | 1 | x 10 | - |
| Red | 2 | 2 | x 100 | - |
| Orange | 3 | 3 | - | - |
| Yellow | 4 | 4 | - | 6.3V |
| Green | 5 | 5 | - | 16V |
| Blue | 6 | 6 | - | 20V |
| Violet | 7 | 7 | - | - |
| Gray | 8 | 8 | x 0.01 | 25V |
| White | 9 | 9 | x 0.1 | 3V |
| Pink | - | - | - | 35V |

Common Capacitor Working Voltages (DC), By Capacitor Type.

| Ceramic | Electrolytic | Tantalum | Mylar (Polyester) | Mylar (Metal Film) |
| --- | --- | --- | --- | --- |
|  | 10V | 10V |  |  |
| 16V | 16V | 16V |  |  |
|  |  | 20V |  |  |
| 25V | 25V | 25V |  |  |
|  | 35V | 35V |  |  |
| 50V | 50V | 50V | 50V |  |
|  | 63V |  |  |  |
| 100V | 100V |  | 100V |  |
|  | 160V |  |  |  |
|  |  |  | 200V |  |
|  | 250V |  |  | 250V |
|  | 350V |  |  |  |
|  |  |  | 400V | 400V |
|  | 450V |  |  |  |
| 600V |  |  |  |  |
|  |  |  |  | 630V |
| 1000V |  |  |  |  |